Name: Y.Anjaneyulu

Reg.No.: 192412355

Q.  End-to-End 5G Multirate Receiver Simulation

Simulate an end-to-end 5G receiver employing multi-rate DSP techniques and evaluate signal quality. (25) (BL5) (WK4)

Given Data:

- Input Sampling Rate = 245.76 MHz

- Decimation Factor = 8

- OFDM Signal Bandwidth = 100 MHz

Tasks:

- Generate the received signal.

- Implement filtering.

- Perform decimation.

- Estimate output spectrum.

- Evaluate signal quality.

- Analyze processing efficiency.

Code :

%% End-to-End 5G Multirate Receiver Simulation

% Parameters

Fs_in   = 245.76e6;   % Input sampling rate (Hz)

M       = 8;          % Decimation factor

BW      = 100e6;      % OFDM signal bandwidth (Hz)

Fs_out  = Fs_in / M;  % Output sampling rate after decimation

N       = 2^16;       % Number of samples for simulation

%% 1. Generate the received OFDM-like signal

t = (0:N-1)/Fs_in;

% Simulate a bandlimited OFDM-like signal using random QAM symbols

numCarriers = 1024;

data = randi([0 3], numCarriers, 1); % QPSK symbols

qamSig = qammod(data, 4);

% Place carriers in frequency domain

X = zeros(N,1);

X(1:numCarriers) = qamSig;

rxSig = ifft(X); % Time-domain OFDM-like signal

% Add noise

rxSig = rxSig + 0.01*randn(size(rxSig));

%% 2. Filtering (anti-aliasing filter before decimation)

% Design low-pass filter with cutoff = BW/2

lpFilt = designfilt('lowpassfir', ...

'PassbandFrequency', BW/2/(Fs_in/2), ...

'StopbandFrequency', (BW/2+20e6)/(Fs_in/2), ...

'PassbandRipple', 0.1, ...

'StopbandAttenuation', 60, ...

'DesignMethod', 'equiripple');

filteredSig = filter(lpFilt, rxSig);

%% 3. Perform decimation

decSig = downsample(filteredSig, M);

%% 4. Estimate output spectrum

figure;

pwelch(decSig, hamming(2048), 1024, 4096, Fs_out);

title('Output Spectrum after Decimation');

%% 5. Evaluate signal quality (SNR)

signalPower = bandpower(decSig);

noisePower  = bandpower(decSig - rxSig(1:length(decSig)));

SNR = 10*log10(signalPower/noisePower);

fprintf('Estimated SNR after processing: %.2f dB\n', SNR);

%% 6. Analyze processing efficiency

fprintf('Input samples: %d\n', length(rxSig));

fprintf('Output samples after decimation: %d\n', length(decSig));

fprintf('Reduction factor: %.2f\n', length(rxSig)/length(decSig));

Output: