Problem Restatement

A sampling rate converter is supposed to convert 44.1 kHz → 48 kHz using interpolation factor L=160and decimation factor M=147. But the problem statement says it was implemented as L=147, M=160, which gave the wrong output rate of 40.5 kHz.

1. Check the incorrect computation

Formula for output sampling rate:

Fout=Fin⋅LM

Using the wrong values:

Fout=44.1⋅147160=40.5 kHz

So the error is that L and M were swapped.

2. Correct computation

With the correct values:

Fout=44.1⋅160147

Fout≈48.0 kHz

✅ Final Answer:

- Computational error: Interpolation and decimation factors were swapped.

- Correct output rate: 48 kHz.